Downgrading the Xbox360

Introduction

During the boot process the 360 performs several checks on the contents of the flash to prevent us from downgrading it and exploiting vulnerable versions of the Hypervisor to obtain the “per box” CPU keys. Using the Infectus and some simple software tools we can defeat one of the checks (the 2BL authentication hash) and boot the original launch day version of the XBox software.
Downgrading is a two stage process, first a “downgrader” flash image is created by combining data from a dump of your current, working, XBox flash and a set of original 2.0.1888 files (these can be found in “the usual places”). The Kernel is to be downgraded to 2.0.1888 and it is necessary to recreate the 2.0.1888 Filesystem.

Once an image has been created and loaded into the 360’s flash the Downgrader application is used to search for a “good” 2BL hash that satisfies the 360 and allows us to load the old firmware. You may then update your console to a vulnerable version (4532 or 4548) and obtain the CPU keys for your Xbox. 
Before You Begin…
You will need the following:

1) Xbox360 with Infectus chip and addon interface installed, infectus software version XXX
2) The Degraded Application (Degraded.exe) in a directory on your HDD
3) The Downgrade Tool (iDGTool.exe, Infectus.dll and SiUSBXp.dll) in a directory on your HDD

4) The 360 NAND Tool, version 0.87

5) The contents of the original 2.0.1888 filesystem, unpack the file 1888.FS.rar to a directory on your hard drive (get it in “the usual places”).

6) A dump of your Xbox Flash obtained using the Infectus chip.

Items 2, 3 and 4 can be downloaded from the infectus website. 

Optional: Remove R6T3 resistor. Not for the average user, the resistor is small and difficult to handle, you may damage your XBox. Blowing a new fuse is not a problem, but if you plan to upgrade several time for experiments and you wish to remove it do so with care.

Installing the Addon

[image: image1.jpg]Pxboxg360)DowngradedDiagramidBottom]sidel
oyl [need]tolconnectfonel3vslandlground] Do)
ﬁm@mmmmmmgm





You should follow the normal Infectus install with addition of a wire from point 0 on the Infectus PCB to the JTAG Reset point (marked REST). The POST port connections 0-7 should be connected via the infectus Addon (or homebrew level shifter) to Infectus Pins 10-17

Creating the “Downgradable” Image

To create the “downgrader” image start the Degraded tool:

[image: image2.jpg]Degraded

Status

CB Version 1868 LDV 1
CD Version 1828
CE Version 1868
CF(0) Version 4532 LDV 2

Close. Buid Downgrader Image Setings





The Degraded Application
First, click the “Settings” button and verify the following:
1) 1BL Key is “DD88AD0C9ED669E7B56794FB68563EFA”
2) 1888 File System is the directory where you unpacked the 1888.FS.rar file.

3) File System Start should be set to 39 
[image: image3.jpg]1BLKey

[FOBBADOCSEDEedE 75567 94FGEGEAERA

1838 Fil System

D \Downgrader\ 1883 F51

File System Stait

o =





The Settings Dialog

Next, load your flash dump by clicking the “…” button and selecting the file, the Degraded tool will display information extracted from the dump. 
To create the “downgrader” image click on the “Build Downgrader Image” button and select a directory and filename to save the “downgrader”. Exit the Degraded tool. 
You should now load the “downgrader” image into your 360’s flash using the Infectus tools.

Searching for the 2BL Hash
To run the downgrade tool you should copy the “downgrader” image generated previously to the directory on your HDD where the Downgrade Tool was copied. 
1. Start a command prompt and ‘cd’ to the directory where the Downgrade Tool is located.

2. Run the Downgrade Tool at the command prompt

3. Power on the Xbox and wait for the RRoD

4. Press a key to begin the process.

5. Wait approximately 1 hour while the search algorithm does its thing
The Downgrade Tool requires a minimum of 2 command line parameters:

iDGTool SS File
Where SS is the number of attempts to measure the hash timing and should be set to 1

and File is the “downgrader” image generated previously. The Downgrade Tool will examine the “downgrader” image and begin the process of searching for the correct CB hash. The Downgrade Tool outputs information as it runs: 
"Downgrader" File 1888G.raw

Pairing Data 0x38695E 02

H[16 00000000000000000000000000000000]

Initial Hash:

H[0 XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX]

Turn on your Xbox, press any key when the RRoD starts 
H[0 00XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17846 A 17844 D 3 : 0 NEXT

H[0 01XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17843 A 17843 D 0 : 0 NEXT

H[0 02XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17846 A 17844 D 3 : 0 NEXT

H[0 03XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17847 A 17844 D 3 : 0 NEXT

H[0 04XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17839 A 17843 D 0 : 0 NEXT

H[0 05XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17842 A 17843 D 0 : 0 NEXT

H[0 06XXXXXXXXXXXXXXXXXXXXXXXXXXXXXX] M 17847 A 17843 D 4 : 0 NEXT
The information displayed is:

1. The index into the hash currently being tested and the hash written to the flash. 

2. The timing measurement for this hash.
3. The average timing measurement for this hash index

4. The difference between the measurement and the average.

5. A “confidence” figure

6. The search algorithms decision on the candidate byte.
The process will continue until the correct hash is found and then stop and report “BOOT”

Unfortunately, things go wrong sometimes and the process may be interrupted. 
Very occasionally a correct byte is missed, the search algorithm will cycle through all 256 possible candidate bytes until it finds the correct one or it is stopped. 
If the process is interrupted before it is completed you can restart at that point with 2 extra command line parameters:
iDGTool SS File X YY…YY

Where X is the number of known hash bytes and YY…YY are the bytes themselves. If the last guess before the process was interrupted is:  

H[8 4F700DF50BB8B8EF22XXXXXXXXXXXXXX] M 17933 A 17932 D 1 : 0 NEXT
Then 8 bytes (4F700DF50BB8B8EF) have been found and the command line parameters:

iDGTool 1 1888G.raw 8 4F700DF50BB8B8EF
Will restart the process at the point where it was interrupted. 

"Downgrader" File 1888G.raw

Pairing Data 0x38695E 02

H[16 00000000000000000000000000000000]

Initial Hash:

H[8 4F700DF50BB8B8EFXXXXXXXXXXXXXXXX]

Turn on your Xbox, press any key when the RRoD starts 
Even more occasionally an incorrect candidate byte will be selected, this will be quite obvious for 2 reasons

1. The algorithm will loop for ever, never finding another correct candidate.

2. There will be a large number of large negative measurements and the average will fall by 10 to 11 units.

Interrupt the process and restart it, use the command line options to restart the process. In this case we want to go back 1 byte in the hash and try to guess it again. Reduce the number of guessed hash bytes by 1 (the X in the line H[X …) and restart.
Finally

Your Xbox should now boot and prompt you to select your language etc etc. You should obtain and apply an update that contains a vulnerable kernel (4532 or 4548) and obtain your CPU fuse data. 
There is a final step to the process to cleanup and stealth the downgrade. The CB section will still contain a “suspicious” version lockdown number and once the CPU fuse data is available this should be fixed using the NAND flash dump tool. You can do this one of 2 ways:

1) Patch the CB version lock down to 0 in your new, vulnerable image or, better 
2) Increment 1 or both (if both are present) of the the CF lock down counters by 1 in your original flash image (the 4532 update will blow another eFuse). Reflash your Xbox.
Unless you have applied the maximum number of updates (and blown as many eFuses as possible) removing R6T3 is NOT recommended for the average user. It’s small and difficult to work with and damage may result. 

Known Problems

Sometimes the Downgrader Tool will hang when it starts, this appears to be due to the Infectus being in a strange state. Power off the Xbox, remove the USB cable from the Infectus, remove and then replace the Xbox power cable, replace the Infectus USB cable and try again.

